REPUBLIC OF ARMENIA

“SCIENTIFIC RESEARCH INSTITUTE OF ENERGY”

CLOSED JOINT-STOCK COMPANY

(“INSTITUTE OF ENERGY” CJSC)

Approved by

“Institute of Energy” CJSC

General Director

________________ R. Yeghoyan

«____»_________________2018

REPORT OF TASKS 2 AND 3

Industrial Solar Energy Program

Assessment of Solar PV Development in Armenia

AGREEMENT No. SPP-CS-2/2017

between

Armenia Renewable Resources And Energy Efficiency Fund

and

Scientific Research Institute of Energy CJSC, Armenia

EI - 09/17

Project Manager- Ph.D., Associate Professor T. Gnuni

Yerevan, 2018

Performers:	V. Sargsyan

S. Shatvoryan

A. Tsughunyan

Content

1. INTRODUCTION	6

2. UPGRADING AND DATA INPUT IN THE MONITORING AND ASSESSMENT STRUCTURE OF NEEAP-2	7

3. TRAINING/CAPACITY BUILDING OF THE RA MEINR STAFF FOR COMPILATION OF THE ENERGY BALANCE	16

3.1. Electricity Balance	16

3.2. Natural Gas Balance	17

3.3. Thermal energy balance	18

3.4. Oil product balance	20

3.5. Coal Balance	21

3.6. Balance of wood and other biofuels	22

3.7. Renewable energy balance	23

4. CONCLUSION	25

ANNEX	26

ABBREVIATIONS

| CNG FS | Compressed natural gas filling station |
| --- | --- |
| NSS | National Statistical Service |
| CN FEA | Commodity Nomenclature of Foreign Economic Activity |
| HVEN | High Voltage Electric Networks |
| EU | European Union |
| NEEAP | National Energy Efficiency Action Plan |
| EEAP-2 | Second phase of Energy Efficiency Action Plan for 2017-2018 |
| MEINR | Ministry of Energy Infrastructures and Natural Resources |
| EB | Energy Balance |
| AR | Artsakh Republic |
| HPP | Hydro Power Plant |
| ENA | Electric Network of Armenia |
| PSRC | Public Services Regulatory Commission |
| RA | Republic of Armenia |
| WPP | Wind Power Plant |
| R2E2 | Armenia Renewable Resources And Energy Efficiency Fund |
| M&A | Monitoring and assessment |
| IEA | International Energy Agency |
| TPP | Thermal Power Plant |
| RF | Russian Federation |
| UGS | Underground gas storage |
| EDRC | Economic Development Research Center |
| OECD | Organization for Economic Cooperation and Development |
| ILCS | Integrated Living Conditions Survey |
| CJSC | Closed Joint Stock Company |
| PV | Photovoltaic |

Measurement units

| mln. | million |
| --- | --- |
| km | kilometer |
| t | ton |
| toe | ton oil equivalent |
| ktoe | kiloton oil equivalent (1000 toe) |
| m3 | cubic meter |
| mln. m3 | million cubic meter |
| W | Watt |
| kW | kiloWatt (103 W) |
| MW | MegaWatt (106 W) |
| Wh | Watt*hour |
| kWh | kiloWatt*hour (103 Wh) |
| MWh | MegaWatt*hour (106 Wh) |
| GWh | GigaWatt*hour (109 Wh) |
| J | Joule |
| MJ | MegaJoule (106 J) |
| TJ | TeraJoule (1012 J) |

# 1. INTRODUCTION

The given report was developed by the Scientific Research Institute of Energy CJSC based on the agreement concluded with the Armenia Renewable Energy and Energy Efficiency Fund in frame of the Industrial Solar Energy Program.

2016 Energy Balance of the Republic of Armenia was developed in line with the aforesaid Agreement. Energy balance is an important tool for the study and development of the national policy for the country’s energy field. It enables the assessment of the energy security of the country, the degree of energy supply degradation, the development of renewable energy, the amounts of greenhouse gas emission as well as to analyze the progress in the implementation of the energy efficiency projects, etc.

The given report includes information on the following Tasks of the Agreement:

- Task 2 - Upgrading and data input in the monitoring and assessment structure of NEEAP-2

- Task 3 – Capacity building in the RA MEINR for the development of the energy balance.

Work was carried out using the “Monitoring Framework” Excel program developed by the Economic Development Research Center (EDRC).

# 2. UPGRADING AND DATA INPUT IN THE MONITORING AND ASSESSMENT STRUCTURE OF NEEAP-2

This chapter represents the data sources used for the assessment of NEEAP-2 and the results obtained.

Data Sources

Data on the RA macroeconomic indicators were collected from the official website of RA PSRC. Sources for the RA energy indicators were the energy balances developed by RA NSS in 2014, Economic Development Research Center (EDRC) in 2015 and by the Scientific Research Institute of Energy in 2016.

RA macroeconomic and other related indicators as well as the monitoring results are presented in the tables below.

| Macroeconomic and other related indicators |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| N | Indicator’s name | Measurement unit | Source | Publication or link | 2014 | 2015 | 2016 |
| 1 | GDP, nominal | Billion AMD | RA NSS | http://armstat.am/am/?nid=202 | 4,828.6 | 5,043.6 | 5,079.7 |
| 2 | Industry and construction sectors’ added value, nominal | Billion AMD | RA NSS | http://armstat.am/am/?nid=202 | 1,220.2 | 1,294.1 | 1,256.1 |
| 3 | Transport sector’s added value, nominal | Billion AMD | RA NSS | http://armstat.am/am/?nid=202 | 144.5 | 120.8 | 129.9 |
| 4 | Households’ added value, nominal | Billion AMD | RA NSS | http://armstat.am/am/?nid=202 | 2.8 | 2.7 | 2.8 |
| 5 | Agriculture sector’s added value, nominal | Billion AMD | RA NSS | http://armstat.am/am/?nid=202 | 872.6 | 869.0 | 809.7 |
| 6 | Service sector’s added value, nominal | Billion AMD | RA NSS | http://armstat.am/am/?nid=202 | 2,142.1 | 2,298.9 | 2,464.7 |
| 7 | Added value (Gross, at basic prices) | Billion AMD | RA NSS | http://armstat.am/am/?nid=202 | 4,282.3 | 4,496.4 | 4,571.2 |
| 8 | Financial Intermediate Services Indirectly Measured (FISIM) | Billion AMD | RA NSS | http://armstat.am/am/?nid=202 | -100.0 | -89.2 | -92.1 |
| 9 | Stable population, average annual | Thousand people | RA NSS | http://armstat.am/am/?nid=202 | 3,010.6 | 2,998.6 | 2,986.1 |
| 10 | GDP per capita | Million AMD | RA NSS | http://armstat.am/am/?nid=202 | 1,603.9 | 1,682.0 | 1,701.1 |
| 11 | Gross domestic consumption of energy | ktoe | RA NSS, EDRC | 2014, 2015 and 2016 RA Energy Balances | 3,120.4 | 3,100.1 | 3,117.8 |
| 12 | Available amount for final energy consumption | ktoe | RA NSS, EDRC, SRIE | 2014, 2015 and 2016 RA Energy Balances | 2,244.4 | 2,307.0 | 2,118.7 |

| Group 1. Energy efficiency overall indicators |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| N | Indicator’s name | Measurement unit | Calculation method | 2015 | 2016 |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | Energy intensity in the economy | ktoe/billion AMD | TPES (ktoe)/ GDP (nominal, billion AMD) | 0.61 | 0.61 |
| 2 | Energy efficiency in the economy | billion AMD/ktoe | GDP (nominal, billion AMD)/ TPES(ktoe) | 1.63 | 1.63 |
| 3 | Energy intensity in the main sectors |  |  |  |  |
| 3.1 | Energy intensity in the industry (including construction) | ktoe/billion AMD | Final consumption for energy purposes in the given sector (ktoe)/ to the added value of the given sector (billion AMD) | 0.24 | 0.25 |
| 3.2 | Energy intensity in the transport | ktoe/billion AMD | Final consumption for energy purposes in the given sector (ktoe)/ to the added value of the given sector (billion AMD) | 5.52 | 4.75 |
| 3.3 | Energy intensity in the households | ktoe/billion AMD | Final consumption for energy purposes in the given sector (ktoe)/ to the added value of the given sector (billion AMD) | 288.21 | 278.32 |
| 3.4 | Energy intensity in the agriculture sector | ktoe/billion AMD | Final consumption for energy purposes in the given sector (ktoe)/ to the added value of the given sector (billion AMD) | 0.05 | 0.05 |
| 3.5 | Energy intensity in the service sector | ktoe/billion AMD | Final consumption for energy purposes in the given sector (ktoe)/ to the added value of the given sector (billion AMD) | 0.21 | 0.13 |
| 4 | Gross domestic consumption of energy | ktoe | Based on the Energy balance | 3,100.1 | 3,117.8 |
| 4.1 | Gross domestic consumption of energy, per capita | toe | TPES (toe)/ stable population (people, average annual) | 1.034 | 1.044 |
| 4.2 | Energy saving in the gross domestic consumption | %, compared to last year | Indicator which is calculated dividing the TPES (ktoe) of the current year by that of the previous year and multiplied by (-1) which is expressed in percentage afterwards. In case of symbol (+), the indicator points out reduction or saving in consumption. | 0.7 | -0.6 |

| 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- |
| 5 | Available capacity for final energy consumption | ktoe | Based on the Energy balance | 2,307.0 | 2,118.7 |
| 5.1 | Available capacity for final energy consumption, per capita | toe | Available amount for final energy consumption (toe)/ stable population (people, average annual) | 0.769 | 0.710 |
| 5.2 | Energy saving in the final consumption | %, compared to last year | Indicator which is calculated dividing available final consumption (ktoe) of the current year by that of the previous year and multiplied by (-1) which is expressed in percentage afterwards. In case of symbol (+), the indicator points out reduction or saving in consumption. | -2.8 | 8.2 |
| 6 | Weighted losses in energy distribution and transmission | %, against the final energy consumption | Energy losses in distribution and transmission (ktoe)/ available amount for final energy consumption (ktoe) expressed in percentage | 7.8 | 8.5 |
| 6.1 | Weighted losses of the electricity | %, against the final electricity consumption | Electricity losses in distribution and transmission (ktoe)/ available amount for final electricity consumption (ktoe) expressed in percentage | 15.1 | 13.2 |
| 6.2 | Weighted losses of the natural gas | %, against the final natural gas consumption | Natural gas losses in distribution and transportation (ktoe)/ available amount for final natural gas consumption (ktoe) expressed in percentage | 8.5 | 9.8 |

| Group 2. Primary energy production and renewable energy |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| N | Indicator’s name | Measurement unit | Calculation method | 2015 | 2016 |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | Primary production of the nuclear energy | ktoe | Based on the Energy balance | 726.4 | 713.8 |
| 1.1 | Share of nuclear energy primary production against the total gross domestic consumption of energy | % | Indicator which is calculated dividing the primary production amount of the nuclear energy (ktoe) by the TPES which is expressed in percentage afterwards. | 23.4 | 22.9 |
| 2 | Primary production of the coal energy | ktoe | Based on the Energy balance | 0.5 | 0.8 |
| 2.1 | Share of coal energy primary production against the total gross domestic consumption of energy | % | Indicator which is calculated dividing the primary production amount of the coal energy (ktoe) by the TPES which is expressed in percentage afterwards | 0.01 | 0.02 |
| 3 | Primary production of the renewable energy | ktoe | Total of the primary production indicators (ktoe) of hydro, wind, solar and geothermal energies as well as that of the wood and biomass | 382.7 | 344.3 |
| 3.1 | Share of renewable energy primary production against the total gross domestic consumption of energy | % | Indicator which is calculated dividing the primary production amount of the renewable energy (ktoe) by the TPES which is expressed in percentage afterwards | 12.3 | 11.0 |

| 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- |
| 4 | Primary production of the hydro energy | ktoe | Based on the Energy balance | 189.6 | 202.2 |
| 4.1 | Primary production of small hydro power plants | ktoe | Input from the Energy balance | 71.9 | 82.3 |
| 5 | Primary production of the solar energy | ktoe | Based on the Energy balance | 2.5 | 2.3 |
| 5.1 | Primary production of the installed solar PV plants | mln.kWh | Input from Access 6 of the Energy balance | 1.50 | 0.98 |
| 5.2 | Primary production of the installed solar PV plants | ktoe | To convert the primary production indicator of solar PV from mln.kWh to ktoe it is multiplied by 3.6 and divided by 41.868 | 0.13 | 0.08 |
| 5.3 | Area of the installed solar thermal collectors | m2 | Input from Access 6 of the Energy balance | 21,060.0 | 20,000.0 |
| 5.4 | Primary production of the installed solar thermal collectors | ktoe | Calculated in the following way: a. Area is multiplied by the average annual value of the solar energy flux onto the horizontal surface that is 1720 kWh/m2, b. Extracted value is multiplied by the average annual efficiency factor that is 0.75, c. Revealed indicator is multiplied by 3.6 and divided by 418680000 | 2.3 | 2.2 |

| 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- |
| 6 | Primary production of the wind energy | ktoe | Based on the Energy balance | 0.3 | 0.2 |
| 7 | Primary production of the geothermal energy | ktoe | Based on the Energy balance |  |  |
| 8 | Primary production of the fire wood energy | ktoe | Based on the Energy balance | 164.7 | 84.4 |
| 9 | Primary production of other biomass energy | ktoe | Based on the Energy balance | 25.5 | 55.2 |
|  | *Renewable energy includes hydro energy, wind energy, solar energy, geothermal energy, wood and biomass. |  |  |  |  |

| Group 3. Imported energy resources |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| N | Indicator’s name | Measurement unit | Calculation method | 2015 | 2016 |
| 1 | Total imported energy resources | ktoe | Based on the Energy balance | 2,244.4 | 2,209.7 |
| 1.1 | Share of the total imported energy resources against the total gross domestic consumption of energy | % | Indicator which is calculated dividing the total amount of the imported energy (ktoe) by the TPES which is expressed in percentage afterwards | 72.4 | 70.9 |
| 2 | Imported natural gas | ktoe | Based on the Energy balance | 1,872.7 | 1,847.4 |
| 2.1 | Share of the imported natural gas against the total gross domestic consumption of energy | % | Indicator which is calculated dividing the total amount of the imported natural gas (ktoe) by the TPES which is expressed in percentage afterwards | 60.4 | 59.3 |
| 3 | Imported oil products | ktoe | Based on the Energy balance | 346.8 | 331.1 |
| 3.1 | Share of the imported oil products against the total gross domestic consumption of energy | % | Indicator which is calculated dividing the total amount of the imported oil products (ktoe) by the TPES which is expressed in percentage afterwards | 11.2 | 10.6 |
| 4 | Imported electricity | ktoe | Based on the Energy balance | 14.9 | 23.7 |
| 4.1 | Share of the imported electricity against the total gross domestic consumption of energy | % | Indicator which is calculated dividing the imported electricity amount (ktoe) by the TPES which is expressed in percentage afterwards | 0.5 | 0.8 |

Comparison of the indicators selected from RA Energy Balance

| N | Indicator’s name | 2014* | 2015** | 2016*** |
| --- | --- | --- | --- | --- |
| 1 | Energy intensity of the economy, oe/AMD | 0.646 | 0.615 | 0.614 |
| 2 | TPES, ktoe | 3,120.4 | 3,100.1 | 3,117.8 |
| 3 | TPES, per capita, toe | 1.035 | 1.034 | 1.044 |
| 4 | Primary production of renewable energy, ktoe | 207.9 | 382.7 | 344.3 |
| 4.1 | Primary production of hydro energy | 171.3 | 189.6 | 202.2 |
| 4.2 | Primary production of wind energy | 0.3 | 0.3 | 0.2 |
| 4.3 | Primary production of solar energy | 2.4 | 2.5 | 2.3 |
| 4.4 | Primary production of wood energy | 31.9 | 164.7 | 84.4 |
| 4.5 | Primary production of other biomass energy | 2.0 | 25.5 | 55.2 |
| 5 | Share of the hydro energy primary production against the TPES, % | 6.7 | 12.3 | 11.0 |
| 6 | Available amount for final energy consumption, ktoe | 2,244.1 | 2,307.0 | 2,118.7 |
| 7 | Available amount for final energy consumption, per capita, toe | 0.74 | 0.77 | 0.71 |
| 8 | Final consumption for energy purposes, ktoe | 2,215.1 | 2,281.9 | 2,089.3 |
|  | Oil products | 333.2 | 306.2 | 276.8 |
|  | Natural gas | 1,385.9 | 1,292.1 | 1,207.6 |
|  | Electricity | 460.3 | 478.9 | 458.2 |
|  | Fire wood | 31.9 | 171.9 | 84.5 |
| 8.1 | Households, ktoe | 353.6 | 785.8 | 784.6 |
|  | Oil products | 5.1 | 7.5 | 0.66 |
|  | Natural gas | 353.6 | 415.6 | 480.6 |
|  | Electricity | 166.3 | 161.5 | 159.5 |
|  | Fire wood | 31.9 | 171.9 | 83.7 |
| 8.2 | Branches of economy, toe | 1,861.5 | 1,496.1 | 1,304.7 |
|  | Oil products | 328.1 | 298.7 | 276.1 |
|  | Natural gas | 1,032.3 | 876.6 | 727.0 |
|  | Electricity | 294.0 | 317.4 | 298.7 |
|  | Fire wood | 0.0 | 0.0 | 0.8 |

*Source - NSS and MENR of RA, 2016.

**Source – Economic Development Research Center (EDRC), 2016.

***Source – Scientific Research Institute of Energy, 2017.

Description curves of the monitoring results are presented in the Annex of the Report.

# 3. TRAINING/CAPACITY BUILDING OF THE RA MEINR STAFF FOR COMPILATION OF THE ENERGY BALANCE

A capacity building training has been provided for the RA MEINR Chief of the Energy Department, Mr. Hayk Badalyan, Head of the Energy Efficiency and Technical Norms Department, Mr. Vahagn Atayan, Chief Specialist of the Energy Efficiency and Technical Norms Department, Ms. Anna Martirosyan, Senior Specialist of the Energy Efficiency and Technical Norms Department, Ms. Anna Papyan, Chief Specialist of the Renewable Energy Department, Ms. Nazik Margaryan and to the Senior Specialist of the Renewable Energy Department, Ms. Sona Atayan for the development of the energy balance.

Trainees were provided with the manuals on statistics published by the Eurostat, International Energy Agency and the Organization for Economic Co-operation and Development (OECD). Necessary explanations were given on the methodology issued in the manuals.

Trainee fulfilled the collection, monitoring and input of the initial data along with the experts of the “Scientific Research Institute of Energy” CJSC, as well as participated in the preliminary, intermediate and final results discussions together with the RA NNS and MEINR specialists.

The given chapter presents the principles of compilation of the energy balance articles on which the trainees have acquired the needed know-hows.

## 3.1. Electricity Balance

Article 1. Gross Domestic Consumption

Only the electricity import and export data are filled in this section. The line of “primary production” isn’t completed as it is designed for the primary energy resources and electricity is secondary. There are also no notes in the lines “International aviation bunkers” and “Storage changes” since electricity isn’t stored.

Article 2. Transformation - Receipts

This article isn’t completed for electricity since it is generated as a result of the primary energy resources transformation.

Article 3. Transformation-output

This section is shaped based on the data of the electricity produced by nuclear and thermal power plants.

Article 4. Swap and transfer, return

This point is formed based on the data provided in the balance of the renewable energy resources.

Article 5. Consumption in the energy sector

It summarizes the costs of the own needs of the power stations. It is filled according to the data provided on the PSRC’s official website.

Article 6. Losses in distribution and transmission

The section is completed according to the data provided by RA NSS as the sum of the losses occurred in HVEN and ENA networks.

Article 7. Available amount for final consumption

The section is calculated as the sum of Articles 1 to 6.

Article 7.1. Final consumption for non-energy purposes

This article isn’t completed for electricity.

Article 7.2. Final consumption for energy purposes

The result of the electricity final consumption is compiled in the different sectors (industry, agriculture, services, transport, household, etc.) using the sum of the electricity consumption.

Data on the electricity consumption in different sectors were provided by RA NSS.

Article 7.3. Statistical deviation

It is calculated as the residual result deducting Articles 7.1 and 7.2 from Article 7. This article reflects the level of the possible deviations in the data completions used for the balance compilation.

## 3.2. Natural Gas Balance

Article 1. Gross domestic consumption

The line of the “Primary production” isn’t completed since there is no natural gas extraction in Armenia. “Import” and “Export” lines are filled. “Storage changes” line represents the difference between the gas amounts taken from the pipelines and UGS and that of pumped into there.

Article 2. Transformation – Receipts

This Article summarizes the gas amounts delivered to the natural gas operated thermal power plants (condensation) and to combined cycle power plants (cogeneration). Natural gas here is transformed into electrical and thermal energy.

Article 3. Transformation-output

This point isn’t completed for the natural gas because according to the previous chapter the energy revealed in transformation is reflected in the appropriate lines of electrical and thermal energy.

Article 4. Swap and transfer, return

This section isn’t filled.

Article 5. Consumption in the energy sector

This article outlines the gas consumption used for the own needs.

Article 6. Losses in distribution and transmission

Losses in transmission and distribution systems as well as data on the recovered gas are presented in this section.

Article 7. Available amount for final consumption

The section is calculated as the sum of Articles 1 to 6.

Article 7.1. Final consumption for non-energy purposes

According to RA NSS data there was no gas consumption for non-energy purposes in 2016 as the operation of "Nairit" factory, known as synthetic rubber producer in the region, was ceased in 2008 due to the financial crisis. Thus, this article isn't completed.

Article 7.2. Final consumption for energy purposes

The result of the final consumption is compiled in the different sectors (industry, e, transport, services, household, etc.) using the sum of the natural gas consumption.

Data sources on the consumption of the sectors are presented in the "Data sources" subsection of this Chapter.

Article 7.3. Statistical deviation

It is calculated as a residual result deducting Articles 7.1 and 7.2 from Article 7. This article reflects the level of the possible deviations in the data completions used for the balance compilation.

## 3.3. Thermal energy balance

Article 1. Gross domestic consumption

This section isn't completed since the thermal energy is a secondary reserve and there is no import, export and storage of the thermal energy in Armenia.

Article 2. Transformation – Receipts

This article isn't filled for the thermal energy.

Article 3. Transformation-output

The data provided by RA NSS on the thermal energy produced by the cogeneration plants have been used for the formation of this article.

The Biogas operated facility of "Lus Astgh" Ltd. didn't produce electrical and thermal energy in 2016.

Amounts of the thermal energy produced on the solar technology have been determined by the expert assessment methodology described in the section "Renewable Energy Carriers Balance" and agreed with the R2E2.

Article 4. Swap and transfer, return

This section isn’t filled.

Article 5. Consumption in the energy sector

Taking into account the low capacities of cogeneration technologies it has been estimated that consumption of their own needs amounts to 5% of the production. There is no thermal energy consumption for the own needs of solar technologies.

Article 6. Losses in distribution and transmission

Given the poor isolation condition of the heat supply pipelines losses in the distribution amounts to 62.2% of the energy produced by "Armruscogeneration" CJSC with own needs consumption of 5.4%. Thermal energy losses on the solar technologies are 3-7% depending on the outside temperature and the insulation quality of the hot water supply pipes. For the balance compilation, these losses were considered as 5%.

Article 7. Available amount for final consumption

The section is calculated as the sum of Articles 1 to 6.

Article 7.1. Final consumption for non-energy purposes

This article isn't completed for the thermal energy.

Article 7.2. Final consumption for energy purposes

Thermal energy generated at the cogeneration plants is consumed in the households. Heat energy supplied from the cogeneration plants to the stores, workshops, offices and other service organizations located in the residential buildings has been assigned to households.

According to the experts assessments thermal energy produced by solar water heaters is proportionally consumed in households and service sectors.

Article 7.3. Statistical deviation

It is considered as a residual result deducting Articles 7.1 and 7.2 from Article 7. This article reflects the level of the possible deviations in the data completions used for the balance compilation.

## 3.4. Oil product balance

Article 1. Gross domestic consumption

Since there is no oil extraction or refinery in Armenia, the line "Primary Production" isn’t completed. Import and export amounts of the oil products are presented. Since there is no domestic aviation in Armenia and the country's territory is relatively small the volumes of the imported aviation petrol and kerosene are shifted to "International Aviation Bunker".

Article 2. Transformation – input

This article isn't filled for the oil products.

Article 3. Transformation-output

The section isn’t completed for the oil products.

Article 4. Swap and transfer, return

This section isn’t filled for the oil products.

Article 5. Consumption in the energy sector

This article is not completed for oil products.

Article 6. Losses in distribution and transmission

Losses of the oil products in the transportation are negligible and registration is missing. Thus, this section isn’t completed for the oil products.

Article 7. Available amount for final consumption

The section is calculated as the sum of Articles 1 to 6.

Article 7.1. Final consumption for non-energy purposes

Amounts of the mazut, bitumen, special types of gasoline, lubricants, paraffins and other petroleum products consumed for non-energy purposes are presented in this article.

Article 7.2. Final consumption for energy purposes

Data on the types and consumption amounts of the oil products in the industrial sector have been provided by RA NSS.

Information on the gasoline consumption is presented in the transport sector.

Consumption amounts of the diesel fuel in the agriculture sector are based on the sowing and total harvesting areas of the agricultural crops as well as the data on the fuel consumption in the agriculture sector.

Article 7.3. Statistical deviation

It is considered as a residual result deducting Articles 7.1 and 7.2 from Article 7. This article reflects the level of the possible deviations in the data completions used for the balance compilation.

## 3.5. Coal Balance

Article 1. Gross domestic consumption

The line "Primary Production" is completed for the extracted lignites and peat. Import and export amounts of the coal are presented in this article.

Article 2. Transformation – Receipts

This section isn't filled since there is no coal cleaning technologies in Armenia.

Article 3. Transformation-output

The section isn’t completed.

Article 4. Swap and transfer, return

This section isn’t filled.

Article 5. Consumption in the energy sector

This article isn’t completed.

Article 6. Losses in distribution and transmission

Losses of the coal in the transportation are negligible and registration is missing. Thus, this section isn’t completed for the coal.

Article 7. Available amount for final consumption

The section is calculated as the sum of Articles 1 to 6.

Article 7.1. Final consumption for non-energy purposes

Peat is consumed in Armenia for non-energy purposes as a fertilizer.

Article 7.2. Final consumption for energy purposes

According to RA NSS data small quantity of the coal was consumed in the construction subsector in 2016.

Following to the experts estimations the great part of the coal except for the lignites, that is around 80%, is used in the service sector and the remaining percentage in the households. Households also utilize the extracted lignites.

Article 7.3. Statistical deviation

It is considered as a residual result deducting Articles 7.1 and 7.2 from Article 7. This article reflects the level of the possible deviations in the data completions used for the balance compilation.

## 3.6. Balance of wood and other biofuels

Article 1. Gross domestic consumption

The line “Primary production” for the firewood and manure was completed according to the data provided by the households’ Integrated Living Conditions Survey (ILCS) of RA NSS.

The lines “Import” and “Export” were completed using the data provided by RA NSS ILCS.

The line “Storage changes” isn’t filled because the wood and biofuel are considered to be wholly consumed over the year.

Article 2. Transformation – Receipts

This section is completed only in the availability of the biogas power plant for the heat and electricity production. Since the power plant owned by the “Lusakert Biogas Plant” CJSC was out of operation in 2016 this section has no information to complete.

Article 3. Transformation-output

The section isn’t completed.

Article 4. Swap and transfer, return

This section isn’t filled.

Article 5. Consumption in the energy sector

This section is completed only in the availability of the biogas power plant for the heat and electricity production. Since the power plant owned by the “Lusakert Biogas Plant” CJSC was out of operation in 2016 this section has no information to complete.

Article 6. Losses in distribution and transmission

Losses of the wood and other types of biofuel in the transportation are negligible and registration is missing. Thus, this section isn’t completed for the wood and other types of biofuel.

Article 7. Available amount for final consumption

The section is calculated as the sum of Articles 1 to 6.

Article 7.1. Final consumption for non-energy purposes.

Amounts of the manure consumed for non-energy purposes in 2016 have been estimated through the expert methodology.

All the imported liquid biofuel also was consumed for non-energy purposes.

Article 7.2. Final consumption for energy purposes

Data on the consumption of the wood and other types of biofuel in the industrial sector have been provided by RA NSS.

Information about the consumption of the wood and biofuel in the transport sector of Armenia isn’t available.

Estimations on the consumption of the firewood and other types of biofuel in the agriculture and service sectors for the energy purposes aren’t available.

Data on the amounts of the consumed firewood and manure in the households have been provided by RA NSS.

Article 7.3. Statistical deviation

It is considered as a residual result deducting Articles 7.1 and 7.2 from Article 7. This article reflects the level of the possible deviations in the data completions used for the balance compilation.

## 3.7. Renewable energy balance

Article 1. Gross domestic consumption

The line “Primary production” for the WPP is completed using the data posted on the official website of PSRC, and as for the solar, considering the installed capacities of PV plants and the total area of water heating systems.

Article 2. Transformation – Receipts

This article isn’t completed.

Article 3. Transformation-output

The section isn’t completed.

Article 4. Swap and transfer, return

The observed energy carriers are fully transformed. Hydro, wind and PV reserves are converted to power and that of the solar water heaters to heat energy. Data on the electricity produced in hydro and wind power plants are presented according to the information posted on PSRC’s official website. Information about the solar energy transformation is based on the installed capacities of PV plants and on the total surface of the water heating systems.

Article 5. Consumption in the energy sector

This section isn’t completed.

Article 6. Losses in distribution and transmission

This article is not filled.

Article 7. Available amount for final consumption

The section is calculated as the sum of Articles 1 to 6.

Article 7.1. Final consumption for non-energy purposes.

This section is not completed for the observed energy carriers.

Article 7.2. Final consumption for energy purposes

Observed energy resources have been transformed and presented in form of the electric and heat energy.

Article 7.3. Statistical deviation

It is considered as a residual result deducting Articles 7.1 and 7.2 from Article 7. This article reflects the level of the possible deviations in the data completions used for the balance compilation.

# 4. CONCLUSION

During the task implementation:

- 2016 Armenia Energy Balance was being compiled in the formats of Eurostat and International Energy Agency,

- Upgrading, collection and input of the data of NEEAP-2 monitoring and assessment structure were being carried out,

- Training and capacity building of the RA MEINR staff were being accomplished for the development of the Energy Balance.

# ANNEX