REPUBLIC OF ARMENIA

“SCIENTIFIC RESEARCH INSTITUTE OF ENERGY”

CLOSED JOINT-STOCK COMPANY

Approved by

“Scentific Research Institute of Energy” CJSC

General Director

___________________R. Yeghoyan

«____»__________________2018թ.

“Compilation of 2017 Energy Balance of the Republic of Armenia and Assessment of the Progress of the Energy Efficiency Program”

FINAL REPORT OF TASK 1

Agreement No. DC 044/18

Between

United Nations Development Programme (Armenia)

And

Scientific Research Institute of Energy CJSC, Armenia

EI - 12/18

Project Manager - Ph.D., Associate Professor T. Gnuni

Yerevan - 2018

Performers:	V. Sargsyan

H. Gnuni

A. Martirosyan

CONTENTS

1. INTRODUCTION	6

2. SIGNIFICANCE OF THE ENERGY BALANCE	8

3. BALANCE COMPILATION AND APPLIED APPROACHES	10

3.1. Electricity Balance	10

3.2. Natural Gas Balance	11

3.3. Thermal energy balance	13

3.4. Oil product balance	13

3.5. Coal Balance	14

3.6. Balance of wood and other biofuels	15

3.7. Renewable energy balance	16

4. CONCLUSION	18

Energy Balance of Armenia for 2017, Eurostat Format, agregate ………………………............................19

Energy Balance of Armenia for 2017, IEA Format, aggregated …………………......................................22

ABBREVIATIONS

| CNG FS | Compressed natural gas filling station |
| --- | --- |
| SC | Statistical Commity |
| CN FEA | Commodity Nomenclature of Foreign Economic Activity |
| HVEN | High Voltage Electric Networks |
| EU | European Union |
| EEAP | Energy Efficiency Action Plan |
| EEAP-2 | Second phase of Energy Efficiency Action Plan for 2017-2018 |
| MEINR | Ministry of Energy Infrastructures and Natural Resources |
| EB | Energy Balance |
| RAr | Republic of Artcakh |
| HPP | Hydro Power Plant |
| AEB | Armenia Energy Balance |
| ENA | Electric Network of Armenia |
| PSRC | Public Services Regulatory CommiSKion |
| RA | Republic of Armenia |
| WPP | Wind Power Plant |
| R2E2 | Armenia Renewable Resources And Energy Efficiency Fund |
| M&A | Monitoring and assessment |
| IEA | International Energy Agency |
| TPP | Thermal Power Plant |
| RF | Russian Federation |
| UGS | Underground gas storage |
| EDRC | Economic Development Research Centre |
| OECD | Organization for Economic Cooperation and Development |
| ILCS | Integrated Living Conditions Survey |
| CJSC | Closed Joint Stock Company |
| PV | Photovoltaic |

Measurement units

| mln. | million |
| --- | --- |
| km | kilometer |
| t | ton |
| toe | ton oil equivalent |
| ktoe | kiloton oil equivalent (1000 toe) |
| m3 | cubic meter |
| mln. m3 | million cubic meter |
| W | Watt |
| kW | kiloWatt (103 W) |
| MW | MegaWatt (106 W) |
| Wh | Watt*hour |
| kWh | kiloWatt*hour (103 Wh) |
| MWh | MegaWatt*hour (106 Wh) |
| GWh | GigaWatt*hour (109 Wh) |
| J | Joule |
| MJ | MegaJoule (106 J) |
| TJ | TeraJoule (1012 J) |

# 1. INTRODUCTION

The given report was developed by the Scientific Research Institute of Energy CJSC (Armenia) based on the agreement (Agreement) concluded with the United Nations Development Program (Armenia).

Development of RA Energy Balance is important for the assessment of the energy security level of the country, the diversification of power supply and the trends in greenhouse gas emissions, as well as for evaluation of the targets of the energy efficiency action plan.

Energy balance is one of the main sources for the collection of the initial data on greenhouse gases (GG) of the “Energy” sector in the national inventory which is the basis for the development and implementation of the measures addressed to the mitigation of the impacts on the environment. Implementation of the mitigation measures is of a great importance under the Paris agreement and its commitments ratified and undertaken by the Republic of Armenia.

The Energy Balances of Armenia for 2015 and 2016 were compiled by the order of R2E2 using the methodology developed by the R2E2 for compilation of the energy balance.

Energy balance is an important tool for the study and development of the national policy for the country’s energy field. It enables the assessment of the energy security level of the country, the diversification of power supply, the development of renewable energy, the amounts of greenhouse gas emission as well as to analyse the progress in the implementation of the energy efficiency projects, etc.

Energy balance is a serious measure in the country for the assessment, documentation and monitoring of the energy efficiency and renewable energy indicators as well as the targets of the Energy Efficiency Action Plan (EEAP-2).

The given report is a reflection of the activities carried out in the development of 2017 Energy Balance under Task 1 of the Agreement.

Based on the energy statistics manuals published by the Eurostat, International Energy Agency and Organization for Economic Cooperation and Development (OECD), as well as on 2006 developing guidelines for the national GHG inventories of the UN experts in the Intergovernmental Panel on Climate Change (IPCC):

- Collection of the official data has been arranged, including the data required for the compilation of  the GHG inventory;

- Business meetings and discussions have been organized to improve data collection, in particular: (a) for the assessment of the consumption volumes of liquid fuels by sectors; (b) for the evaluation of biofuels consumption

- Methodology for the compilation of 2016 Energy Balance was analysed

- Collected data were analysed, the balance indicators were calculated and the initial data were archived

- 2017 Armenia Energy Balance was compiled and presented in Eurostat and International Energy Agency’s formats.   :

Compilation and publication of Armenia Energy Balance is defined by the RA Law on “Energy Efficiency and Renewable Energy”, and by “2014-2020 Action Plan Ensuring the Implementation of the Provisions to the Energy Security Concept of the Republic of Armenia” approved by Decree № 836-N of the RA Government adopted on July 31, 2014 as well as by the Agreement concluded between MEINR and SC of the Republic of Armenia on information exchange.

Methodological base for the compilation of 2017 Armenia Energy Balance was the guideline published by the IEA, Eurostat and Organization for Economic Cooperation And Development (OECD) as well as the “Explanatory Note on Energy Balance of Armenia” developed by the Economic Development Research Centre (EDRC). Principals, main sources of the initial data and the major applied approaches for the formation of the energy balance are reflected within the mentioned Explanatory Note.

Works have been implemented using the «EDRC_Energy_Balance_of_Armenia_for_2015» Excel program that was slightly upgraded by the “Scientific Research Institute of Energy” CJSC in 2017.

# 2. SIGNIFICANCE OF THE ENERGY BALANCE

Energy balance is represented in the form of a standard spreadsheet format, which reflects amount of the energy resources extracted, produced, imported, exported, stored, processed, converted, transported, distributed and used in various sectors during the reporting period. . Energy balance of the country is presented in the Eurostat or IEA standard formats, which comply with the regulation No 1099/2008 of the European Parliament and of the Council of 22 October 2008 on energy statistics.

During the compilation of the energy balance, it is necessary to take into account the flows of energy carriers and all types of the energy by their generation, recycling, transformation, distribution, storage and final consumption cycles, as well as energy costs and possible losses for own needs.

According to the Eurostat requirements energy balances are composed of natural (TJ) units, and as for the IEA standards - conventional units. Conventional unit is the oil equivalent, which 1 ktoe equals to 41.868 TJ.

Collection and processing of the statistical data and compilation of the energy balance are the first steps for the analysis of the energy situation in the country allowing the assessment of the previous trends and formation of the future policy addressed to the development of the energy sector. Data in the energy balance enable the assessment of the energy efficiency level in the different sectors of the economy and households. It should be noted that emissions indicators are closely related to those of energy efficiency.

The role of clear and comprehensive information is constantly increasing and allowing to not only comply with the qualitative conclusions, but also to reveal quantitative assessments for energy policy.

According to the IEA, in order to develop an energy policy it is necessary to have clear information about the final consumption, in particular:

- Information on the driving forces of the energy consumption,

- Information on the current energy consumption situation,

- Information about the responses on the implementation of the policy options.

Different models have been developed that enable the identification of the existing issues and the evaluation of the possible outcomes of the events taken. The "driving force – situation - reaction" is a key model, which was proposed by the Organization for Economic Co-operation and Development (OECD) in 1996.

All this makes the information environment, which is regularly updated and provides a basis for the policy review and correction.

Despite the fact that the electricity produced by renewable sources (wind, solar, etc.) is still quite expensive, their increase in the energy balance can ensure the reasonable level of energy independence of the country and the reduction of greenhouse gas emissions

Development of the energy balance is necessary for:

- Assessment of the country’s energy independence, including the power produced at the renewable energy sources in the energy balance;

- Study of the fuel and energy production and consumption structure;

- Study of the fuel and energy resources demand;

- Analysis of the fuel and energy balance structure for its improvement;

- Definition of the capital investments volumes for the development of the fuel and energy complex in the country;

- Identification of the export and import volumes of fuel and energy resources;

- Assessment of the energy efficiency potential;

- Identification of the greenhouse gases & other emissions volumes and the development of the measures to minimize their negative impacts on the environment, etc.

# 3. BALANCE COMPILATION AND APPLIED APPROACHES

This chapter presents the initial data of the main energy carriers included in the energy balance and the basic approaches applied for compilation of the balance.

## 3.1. Electricity Balance

General information and overview

Electric energy is one of the most developed areas in the economy of Armenia. There are both the traditional sources for electricity production that are NPP, TPP and HPPs, and the alternative sources.   Governmental decisions contributed to the development of some alternative energy projects which include off-grid and licensed solar photovoltaic systems, solar water heaters, wind power monitoring, geothermal energy research, etc. Armenia exports energy to Iran, RAr and to Georgia as well. Electricity import is also arranged from the mentioned countries. Electricity export to Iran is realized on electricity-for-gas swapping agreement and was 85% of the overall exported electricity in 2017. The import from Iran amounted to 75.1 mln. kWh in 2017, which is mainly conditioned by the power system regimes. Electricity export to Georgia is mainly organized in the emergency switched off the 500 kW Caucasian power transmission line feeding the system from Russia and was 127.4 mln. kWh in 2017. Power supply to the Northern parts of Armenia in wet seasons is performed from Georgia in the isolated regime and amounted to 179.8 mln. kWh in 2017. .

In 2017, Armenian NPP produced 2619.6 million kWh electricity which is around 29.2% of the total production.

There are three big thermal power plants in Armenia, two of which although are combined cycle production capacities but operated in condensation mode in 2017. Combined cycle operated power unit of “Yerevan TPP” CJSC, “Hrazdan-5” of “Gazprom Armenia” CJSC and “Hrazdan TPP” OJSC generated 1543.6 million kWh, 992.6 million kWh and 316.9 million kWh electricity in 2017, respectively. Shares of the mentioned plants in the total electricity production accordingly amount to 19.9%, 12.8% and 4.1%. Thus, the shares of “Hrazdan -5” and “Hrazdan TPP” OJSC in the electricity production increased and decreased respectively compared to the generation in 2016.      Some amount of electricity was also produced at small-size combined cycle power plants. Total production of “Yerevan State Medical University after Mkhitar Heratsi” and “Armruscogenaration” CJSC cogeneration sites in 2017 amounted to 18.6 million kWh or 0.24% of the overall production.

Hydro energy of Armenia are presented by two major HPP cascades owned by “International Energy Corporation” CJSC and “Contour Global Hydro Cascade” CJSC, as well as by a number of small HPPs. The HPPs of “International Energy Corporation” and “Contour Global Hydro Cascade” produced 466.0 million kWh and 941.2 million kWh electricity in 2017 respectively, which accordingly amount to 6.0% and 12.1% of the total electricity production and are comparable to the indicators of 2016. According to the RA PSRC information, number of the small HPPs in 2017 was 182 which total installed capacity amounted to 352.9 MW with 861.9 million kWh of annual electricity production. The share of small HPPs in the total electricity production was 11.1% which is comparable to the indicators of 2016. It is predicted that 216 Small HPPs will operate in Armenia by 2020 having 406.6 MW of total installed capacity and 1145 million kWh of annual electricity production.

Small amount of electricity was also produced in the wind and solar photovoltaic power plants. Total production of the wind farms Lori -1 and another one owned by “Arats” Ltd. amounted to 2.09 million kWh in 2017, which was 0.027% of the total electricity production. Introduction of solar PV plants has significantly developed in 2017.   According to the PSRC, electricity produced at PV sites in 2017 amounted to 0.54 million kWh, 0.14 million kWh of which was generated by the off-grid producers with capacity of up to 150 kW.

Electricity is transmitted and distributed by HVEN and ENA CJSCs wherein inevitable technical losses occur. Losses in 2017 amounted to 668.5 million kWh which is 5.3% less than in 2016.

Data Sources

Data on the production and the own needs of each of the plants, as well as on the losses in the transmission and distribution process have been collected from the PSRC’s official website. Information about the electricity consumption by the sectors has been provided by RA SS.

## 3.2. Natural Gas Balance

General Information and overview

Transmission, distribution, storage and sale of the natural gas in the RA domestic market is realized by the “Gasprom Armenia” CJSC. The total length of the main gas pipelines and pipes’ branches operated in the gas transportation system amounts to 1682.2 km.  Transportation of the gas is carried out through the 14902.0 km length gas pipeline. Unused pipelines are in operational reserve mode. The underground gas storage (UGS) allocated in Abovyan is also used for the gas backup owned by “Gasprom Armenia” CJSC, wherein it is possible to store up to 135 million m3 natural gas. UGS has strategic importance since it ensures the reliability of the gas supply in the country while being used to cover the seasonal and peak demand of the gas. In 2017, 57.7 mil.m3 of gas was taken from the gas pipelines and UGS (14.9% more than in 2016), meanwhile, 43.3mil.m3 was pumped into the UGS (68.1% more than in 2016).

Construction of Iran-Armenia gas pipeline allowed the implementation of the second technological access to the Republic of Armenia while ensuring the diversification of the gas supply. According to the pending agreement it is planned to perform gas-for-electricity swapping. A two-circuit 400kV transmission line is now being constructed to ensure the contractual amounts of the gas imported from Iran.

In 2017, natural gas in amounts of 1996.0 millionm3 (6.9% more than in 2016) and 382.7 millionm3 (4.6% more than in 2016) were imported from the Russian Federation and Iran, respectively. Data on the imported gas are published in the annual report of the RA SC and posted on the PSRC’s official website.

Following to the estimations of “Gazprom Armenia” CJSC around 77% of the vehicles park in Armenia operates in compressed natural gas. Currently, Armenia has leading positions among the countries in the world that apply natural gas as an engine fuel.  It is economically advisable compared to the other oil products and leads to the significant reduction of hazardous emissions and green gas in the atmosphere.  Today, there are 373 CNG filling stations in Armenia. Transport sector consumed 477.5 million m3 of natural gas in 2017 which is 2.4 times more than the volume of the gas consumed in the industry for energy purposes.

Data Sources

Data provided by PSRC, MEINR and RA SC served as the information source for the compilation of the natural gas balance. Information about the gas amounts imported and taken from the gas pipelines and UGS as well as data on the gas volumes pumped into the UGS, losses in transportation and distribution systems, transported gas amounts, own needs consumption, recovered gas volumes and consumption in the transportation sector were taken from the RA PSRC’s official website. Data on the natural gas used in the Industrial sector for energy purposes and in separate power plants were provided by the RA SS.   Information about the gas amounts consumed in the other sectors was supplied by the MEINR

## 3.3. Thermal energy balance

General information and overview

Share of the thermal energy in the overall energy balance of Armenia is quite small. Thermal energy in Armenia is generated only for the domestic market.

In the beginning of 1990’s, energy crisis occurred in Armenia caused by the irregular gas supply due to Armenia’s low solvency and regular explosions of the gas pipeline. This was the reason for the collapse of the centralized heat supply systems. After the gas supply rehabilitation, individual equipment was widely used for hot water preparation and heating.

Currently, centralized heat supply in Armenia is implemented by small combined cycle power plants. "Yerevan State Medical University after Mkhitar Heratsi" produces heat energy for its own needs, and "Armruscogenartion" CJSC realizes heat supply to Hovhannisyan, Varuzhan, Isahakyan blocks of Avan administrative area in Yerevan.

There was no heat supply from two major thermal power plants Hrazdan TPP and Yerevan CCGT. There are some small power boiler houses in Armenia designed for the heating of one or several buildings.

Thermal energy sources also include solar water heaters and fruit drying equipment. The share of thermal energy produced on solar equipment is insignificant and has been considered in the section "Renewable Energy Carriers Balance".

The share of thermal energy produced on solar equipment is insignificant and has been considered in the section "Renewable Energy Carriers Balance". There is no data on the thermal energy generated at the cogeneration station of the fund "Yerevan State Medical University after Mkhitar Heratsi". In 2017, amounts of the thermal energy produced at low capacity combined cycle power plants were 30.6 thousand GJ, which is 10% less than in 2016.

There is no reliable information about the individual solar water heaters' surfaces and the volumes of their utilization in different sectors, particularly, in food industry and service sector.

Data sources

The RA PSRC's official website publishes the data only on the electricity produced by the combined cycle and biogas operated power plants. Information about the thermal energy produced by the combined cycle power plants in 2017 was provided by the RA SS.

## 3.4. Oil product balance

General information and overview

Data on the following oil products are presented in the oil product balance:

- Motor fuel types (engine gasoline, diesel fuel, petrol for jet engines, aviation kerosene)

- Liquid oil gases,

- Mazut and oil bitumen,

- Other oil products (other kerosene types, special types of gasoline, lubricants, paraffin and other oil products).

There is no oil extraction in the territory of Armenia and all the oil products are imported. Some types of the imported oil products are used in the limited amounts for the varnish, paints and for the other product types produced in Armenia. Imported bitumen and mazut are utilized for non-energy purposes either.

Engine gasoline is consumed in the transport sector.

In 2017, the main volume of the diesel fuel in amount of 70.2% was consumed in the transport sector. . Industrial sector used around 14.4% of the diesel fuel. Diesel fuel consumed by the industrial sector is utilized to operate different types of the mechanisms (drilling rigs, cranes, telescopic towers and other mechanisms). Major consumers of the industrial sector are the mining industry and non-ferrous metallurgy.

Around 14.9% of the diesel fuel was used by the agriculture sector (tractors, combines and other mechanisms).

Insignificant amounts of the diesel fuel also are consumed by the diesel generators used for the power production. Data on this sector consumption isn’t available.

In 2017, the main volume of liquid petroleum gases, that are 77.5%, were used by the transport sector and exceeds the consumption of 2016. Consumption volumes of the liquid petroleum gases in the industry, service sectors and in the households were 3.1%, 14.5% and 4.9% respectively.

The rest of the oil products are mainly utilized for non-energy purposes.

Data sources

Information on the imports and exports of oil products is extracted from the RA SC official website. RA SC provided data on the volumes of the oil products consumption in the industrial sector as well as information about the sowing and total harvesting areas of the agricultural crops and the expert assessment of the RA Ministry of Agriculture on fuel consumption in the agriculture sector for 2017.

## 3.5. Coal Balance

General information and overview

RA SC data on the imported coke, semi-coke, anthracite, peat and other coal types were used to compile the coal balance.

In 2017, the main volumes of the coal, around 80%, were used by the service sector for non-energy purposes.

There are lignite mines in Jajur and Dilijan: They haven’t got any industrial significance, and according to the experts estimations around 500 families collect the lignites manually. Therefore, consumption of the lignites in 2017 remained at the level fixed in 2016.

A certain amount of the imported coal is used for non-energy purposes, such as peat as fertilizer.

Data Sources

Information on the imports and exports of the coal types is obtained from the official RA SC website. RA SC provided data on the volumes of the coal consumption in the industrial sector. Information about the lignites was revealed using the expert assessments.

## 3.6. Balance of wood and other biofuels

General information and overview

Wood and other types of biofuel are renewable energy carriers.

Types of the biofuels are:

- Industrial waste,

- Solid household waste,

- Solid biomass (including charcoal),

- Biogases,

- Liquid biomass.

Imported liquid biofuel was used for non-energy purposes.

Firewood and timber in Armenia are revealed in the following ways:

- Sanitary deforestation (solid biomass),

- Illegal deforestation (solid biomass),

- Fallen dry wood (solid biomass),

- Wastes of woodworking and furniture manufacturing (industrial wastes),

- Import.

Data on the timber products and firewood used for the energy purposes in the industrial sector are available and it provides SC. It is more difficult to estimate amounts of the timber products and firewood consumed in the households, which also provides SC based on survey.

Biogas technologies are presented by a 0.85 MW power plant operated by “Lusakert Biogas Plant” CJSC as well as by a small number of individual biogas plants. Amounts of the electricity produced by the “Lusakert Biogas Plant” CJSC are posted on the PSRC’s official website. The plant hasn’t been operating in recent years.   Official information on the individual biogas facilities isn’t available. In Armenia, these technologies mainly use manure. Manure is also utilized for the energy purposes as a furnace fuel, mainly for the heating purposes.

Limited amounts of the crude and other solid residues, as well as liquid biofuels are also imported to Armenia.

Data sources

Information about the imports and exports of the wood and biofuel has been provided by RA SS. RA SC has also provided the data on the amounts of the wood and other biofuel used in the industrial and other sectors.

## 3.7. Renewable energy balance

General information and overview

This chapter observes hydro, wind, solar and geothermal types of energy.

Hydro energy is more developed in Armenia than the other renewable energy resources.

Total installed capacity of seven HPPs owned by the “International Energy Corporation” CJSC amounts to 559.4 MW and production in 2017 was 466.0 million kWh.

Total installed capacity of three HPPs owned by “Contour Global Hydro Cascade” CJSC amounts to 405.4 MW and production in 2017 was 941.2 million kWh.

182 small HPPS were operated in 2017 which total installed capacity amounted to 352.9 MW and the production was 861.9 million kWh.

Two wind power plants (WPP) that are Lori-1 and the WPP owned by “Arats”Ltd were in operation in 2017 with the total installed capacities of 2.6 MW and 20kW, respectively. In 2017, the aforesaid WPPs produced 2.1 million kWh electricity in total.

Armenia also has significant potential for solar energy production. Solar energy is represented by solar water heating and PV power plants. In 2017, amounts of the hot water and electricity produced by the solar technologies increased compared to the volumes in 2016 due to the policy implemented by the RA Government. The method of net metering was applied for autonomous consumers in PV sector.

According to PSRC’s official website, electricity exchange among the autonomous producers with up to 150 kW installed capacity amounted to 0.14 million kWh, and delivery from solar PV plants was 0.4mln. kWh. Since only a portion of the electricity produced by up to 150 kW autonomous producers is delivered to the power system the amounts of the primary production at the solar PV systems couldn’t be extracted due to 0.14 million kWh presented in PSRC official website. Total installed capacity of the autonomous PV systems in 2017 amounted to 1.96 MW following to the information provided by RA MEINR.  As the solar time period in Armenia is 1500 h/year with the installations performance of 0.8 the primary production of the autonomous PV producers was 2.35 million kWh. 2.74 million kWh will be revealed by summing up the electricity delivered from the PV plants to this amount. This figure is about 5 times higher than in 2016.

298 solar water heaters were installed in non gasificated  communities of Armenia under the “Energy Efficient” loan program launched in August 2017. The Government of Armenia is implementing a policy of promoting the development of solar water heating technologies. The customs service data on the water heater technologies imported to Armenia in 2017 were analysed. According to the expert estimation implementation of the various initiatives led to around 20% growth in the produced energy compared to 2016.

Share of the solar technologies in the gross domestic consumption of the renewable energy carriers is still too small and amounted to only 0.86% in 2017.

Geothermal energy in Armenia is represented by several pilot installations which haven’t been included in the energy balance due to their insignificant production volumes.

Data sources

Data on the WPP production amounts were taken from the PSRC’s official website. Sources of the information on the solar technologies were RA MEINR and R2E2.

# 4. CONCLUSION

During the task implementation:

- Energy balances of Armenia for 2010, 2011, 2012, 2014, 2015 and 2016 have been analysed,

- Collection of the official data from RA SS, PSRC, MEINR and R2E2 have been accomplished,

- Energy Balance of Armenia for 2017 has been compiled in the formats of Eurostat and International Energy Agency,

- Preliminary Energy Balance of Armenia for 2017 has been discussed with the specialists from RA SC and MEINR.

Energy Balance of Armenia for 2017, Eurostat Format, agregated, (ktoe)

| N | Energy Balance of Armenia for 2017 | Total | Coal | Oil | Natural Gas | Renewables | Nuclear | Heat | Electricity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| 1.1 | Primary production | 1,109.4 | 2.3 |  |  | 322.9 | 784.2 |  |  |
| 1.2 | Imports | 2,399.5 | 1.1 | 394.1 | 1,968.1 | 8.8 |  |  | 27.5 |
| 1.3 | International aviation bunker | -55.3 |  | -55.3 |  |  |  |  |  |
| 1.4 | Exports | -128.7 | -2.3 | -0.3 |  | -2.3 |  |  | -123.8 |
| 1.5 | Stock changes | -31.0 |  | -13.4 | -19.4 | 1.8 |  |  |  |
| 1 | Gross inland consumption | 3,293.9 | 1.1 | 325.1 | 1,948.6 | 331.2 | 784.2 |  | -96.3 |
| 2 | Transformation - inputs | -1,311.7 |  |  | -527.5 |  | -784.2 |  |  |
| 2.1 | Nuclear power stations | -784.2 |  |  |  |  | -784.2 |  |  |
| 2.2 | Thermal power stations (condensation) | -523.2 |  |  | -523.2 |  |  |  |  |
| 2.3 | Combined heat and power stations (cogeneration) | -4.2 |  |  | -4.2 |  |  |  |  |
| 2.4 | Other |  |  |  |  |  |  |  |  |
| 3 | Transformation - outputs | 472.9 |  |  |  |  |  | 0.7 | 472.2 |
| 3.1 | Nuclear power stations | 225.2 |  |  |  |  |  |  | 225.2 |
| 3.2 | Thermal power stations (condensation) | 245.3 |  |  |  |  |  |  | 245.3 |
| 3.3 | Combined heat and power stations (cogeneration) | 2.3 |  |  |  |  |  | 0.7 | 1.6 |
| 3.4 | Other |  |  |  |  |  |  |  |  |
| 4 | Exchanges and transfers, returns |  |  |  |  | -195.5 |  |  | 195.5 |
| 4.1 | Hydro power stations |  |  |  |  | -121.0 |  |  | 121.0 |
| 4.2 | Small hydro power stations |  |  |  |  | -74.1 |  |  | 74.1 |
| 4.3 | Wind power stations |  |  |  |  | -0.2 |  |  | 0.2 |
| 4.4 | Solar power stations |  |  |  |  | -0.2 |  |  | 0.2 |

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | Consumption in the energy sector (for own needs) | -37.3 |  |  | -6.9 |  |  | 0.0 | -30.4 |
| 5.1 | Nuclear power stations | -17.9 |  |  |  |  |  |  | -17.9 |
| 5.2 | Thermal power stations | -9.6 |  |  |  |  |  | 0.0 | -9.5 |
| 5.3 | Hydro power stations | -3.0 |  |  |  |  |  |  | -3.0 |
| 5.4 | Wind power stations | 0.0 |  |  |  |  |  |  | 0.0 |
| 5.5 | Gas transportation | -6.9 |  |  | -6.9 |  |  |  |  |
| 5.6 | Other stations | 0.0 |  |  |  |  |  |  | 0.0 |
| 6 | Losses in distribution and transmission | -181.2 |  |  | -123.3 |  |  | -0.5 | -57.5 |
| 7 | Available for final consumption | 2,236.6 | 1.1 | 325.1 | 1,291.0 | 135.6 |  | 0.2 | 483.5 |
| 7.1 | Final non-energy consumption | 36.0 | 0.0 | 32.9 |  | 3.1 |  |  |  |
| 7.1.1 | Chemical Industry | 0.0 |  | 0.0 |  |  |  |  |  |
| 7.1.2 | Other sectors | 36.0 | 0.0 | 32.9 |  | 3.1 |  |  |  |
| 7.2 | Final energy consumption | 2,200.7 | 1.1 | 292.2 | 1,291.1 | 132.6 |  | 0.2 | 483.5 |
| 7.2.1 | Industry | 327.9 |  | 20.0 | 165.6 | 0.0 |  |  | 142.3 |
| 7.2.1.1 | Iron and steel | 18.0 |  | 0.0 | 11.9 |  |  |  | 6.1 |
| 7.2.1.2 | Chemical and petrochemical (including petrochemicals) | 3.3 |  | 0.0 | 2.1 |  |  |  | 1.1 |
| 7.2.1.3 | Non-ferrous metals | 45.8 |  | 4.5 | 20.1 | 0.0 |  |  | 21.1 |
| 7.2.1.4 | Non-metallic minerals | 83.8 |  | 0.5 | 71.6 |  |  |  | 11.7 |
| 7.2.1.5 | Transport equipment |  |  |  |  |  |  |  |  |
| 7.2.1.6 | Machinery | 2.6 |  | 0.0 | 0.8 | 0.0 |  |  | 1.7 |
| 7.2.1.7 | Mining and quarrying | 89.6 |  | 14.3 | 6.8 | 0.0 |  |  | 68.5 |
| 7.2.1.8 | Food, beverages and tobacco | 66.3 |  | 0.1 | 44.0 |  |  |  | 22.3 |
| 7.2.1.9 | Paper, pulp and printing | 5.3 |  | 0.0 | 3.5 |  |  |  | 1.8 |
| 7.2.1.10 | Wood and wood products | 0.1 |  |  | 0.1 | 0.0 |  |  | 0.1 |
| 7.2.1.11 | Textiles and leather | 1.5 |  | 0.0 | 0.4 | 0.0 |  |  | 1.1 |
| 7.2.1.12 | Construction | 6.5 |  | 0.4 | 3.4 |  |  |  | 2.7 |
| 7.2.1.13 | Non-specified (Industry) | 5.1 |  | 0.0 | 1.0 |  |  |  | 4.1 |

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7.2.2 | Transport | 653.6 |  | 249.8 | 395.1 |  |  |  | 8.7 |
| 7.2.2.1 | Rail, metro, other electric transport | 6.3 |  |  |  |  |  |  | 6.3 |
| 7.2.2.2 | Road | 644.9 |  | 249.8 | 395.1 |  |  |  |  |
| 7.2.2.3 | Aviation | 1.7 |  |  |  |  |  |  | 1.7 |
| 7.2.2.4 | Other (Transport) | 0.7 |  |  |  |  |  |  | 0.7 |
| 7.2.3 | Households | 812.2 | 0.2 | 0.9 | 514.2 | 132.5 |  | 0.2 | 164.0 |
| 7.2.4 | Agriculture | 34.1 |  | 20.5 |  |  |  |  | 13.6 |
| 7.2.5 | Services | 372.9 | 0.9 | 1.0 | 216.2 |  |  |  | 154.8 |
| 7.3 | Statistical differences | -0.1 | 0.0 | 0.0 | -0.1 | 0.0 |  |  | 0.0 |

Energy Balance of Armenia for 2017, IEA Format, agregated, (ktoe)

| N | Energy Balance of Armenia for 2017, IEA | Total | Coal | Oil | Natural Gas | Renewables | Nuclear | Heat | Electricity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| 1.1 | Production | 1,109.4 | 2.3 |  |  | 322.9 | 784.2 |  |  |
| 1.2 | Imports | 2,399.5 | 1.1 | 394.1 | 1,968.1 | 8.8 |  |  | 27.5 |
| 1.3 | International aviation bunker | -55.3 |  | -55.3 |  |  |  |  |  |
| 1.4 | Exports | -128.7 | -2.3 | -0.3 |  | -2.3 |  |  | -123.8 |
| 1.5 | Stock changes | -31.0 |  | -13.4 | -19.4 | 1.8 |  |  |  |
| 1 | TPES (Total primary energy supply) | 3,293.9 | 1.1 | 325.1 | 1,948.6 | 331.2 | 784.2 |  | -96.3 |
| 2 | Transfers |  |  |  |  |  |  |  |  |
| 3 | Statistical differences | -0.1 |  | 0.0 | -0.1 | 0.0 |  |  | 0.0 |
| 4 | Transformation processes | -838.8 |  |  | -527.5 | -195.5 | -784.2 | 0.7 | 667.7 |
| 4.1 | Electricity plants | -559.0 |  |  |  | -195.5 | -784.2 |  | 420.8 |
| 4.1.1 | Nuclear power stations | -559.0 |  |  |  |  | -784.2 |  | 225.2 |
| 4.1.2 | Hydro power stations |  |  |  |  | -121.0 |  |  | 121.0 |
| 4.1.3 | Small hydro power stations |  |  |  |  | -74.1 |  |  | 74.1 |
| 4.1.4 | Wind power stations |  |  |  |  | -0.2 |  |  | 0.2 |
| 4.1.5 | Solar power stations |  |  |  |  | -0.2 |  |  | 0.2 |
| 4.2 | Thermal power stations (condensation.) | -277.9 |  |  | -523.2 |  |  |  | 245.3 |
| 4.3 | Combined heat and power stations (cogeneration) | -1.9 |  |  | -4.2 |  |  | 0.7 | 1.6 |
| 4.4 | Other |  |  |  |  |  |  |  |  |
| 5 | Consumption in the energy sector (for own needs) | -37.3 |  |  | -6.9 |  |  | 0.0 | -30.4 |
| 5.1 | Nuclear power stations | -17.9 |  |  |  |  |  |  | -17.9 |
| 5.2 | Termal power stations | -9.6 |  |  |  |  |  | 0.0 | -9.5 |
| 5.3 | Hydro power stations | -3.0 |  |  |  |  |  |  | -3.0 |
| 5.4 | Wind power stations | 0.0 |  |  |  |  |  |  | 0.0 |
| 5.5 | Gas transportation system | -6.9 |  |  | -6.9 |  |  |  |  |
| 5.6 | Other stations | 0.0 |  |  |  |  |  |  | 0.0 |

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | Losses in distribution and transmission | -181.2 |  |  | -123.3 |  |  | -0.5 | -57.5 |
| 7 | Total final consumption | 2,236.6 | 1.1 | 325.1 | 1,291.0 | 135.6 |  | 0.2 | 483.5 |
| 7.1 | Final energy consumption | 2,200.7 | 1.1 | 292.2 | 1,291.1 | 132.6 |  | 0.2 | 483.5 |
| 7.1.1 | Industry | 327.9 |  | 20.0 | 165.6 | 0.0 |  |  | 142.3 |
| 7.1.1.1 | Iron and steel | 18.0 |  | 0.0 | 11.9 |  |  |  | 6.1 |
| 7.1.1.2 | Chemical and petrochemical (including petrochemicals) | 3.3 |  | 0.0 | 2.1 |  |  |  | 1.1 |
| 7.1.1.3 | Non-ferrous metals | 45.8 |  | 4.5 | 20.1 | 0.0 |  |  | 21.1 |
| 7.1.1.4 | Non-metallic minerals | 83.8 |  | 0.5 | 71.6 |  |  |  | 11.7 |
| 7.1.1.5 | Transport equipment |  |  |  |  |  |  |  |  |
| 7.1.1.6 | Machinery | 2.6 |  | 0.0 | 0.8 | 0.0 |  |  | 1.7 |
| 7.1.1.7 | Mining and quarrying | 89.6 |  | 14.3 | 6.8 | 0.0 |  |  | 68.5 |
| 7.1.1.8 | Food, beverages and tobacco | 66.3 |  | 0.1 | 44.0 |  |  |  | 22.3 |
| 7.1.1.9 | Paper, pulp and printing | 5.3 |  | 0.0 | 3.5 |  |  |  | 1.8 |
| 7.1.1.10 | Wood and wood products | 0.1 |  |  | 0.1 | 0.0 |  |  | 0.1 |
| 7.1.1.11 | Textiles and leather | 1.5 |  | 0.0 | 0.4 | 0.0 |  |  | 1.1 |
| 7.1.1.12 | Construction | 6.5 |  | 0.4 | 3.4 |  |  |  | 2.7 |
| 7.1.1.13 | Non-specified (Industry) | 5.1 |  | 0.0 | 1.0 |  |  |  | 4.1 |
| 7.1.2 | Transport | 653.6 |  | 249.8 | 395.1 |  |  |  | 8.7 |
| 7.1.2.1 | Rail, metro, other electric transport | 6.3 |  |  |  |  |  |  | 6.3 |
| 7.1.2.2 | Road | 644.9 |  | 249.8 | 395.1 |  |  |  |  |
| 7.1.2.3 | Aviation | 1.7 |  |  |  |  |  |  | 1.7 |
| 7.1.2.5 | Other(Transport) | 0.7 |  |  |  |  |  |  | 0.7 |
| 7.1.3 | Other sectors | 1,219.1 | 1.1 | 22.4 | 730.4 | 132.5 |  | 0.2 | 332.5 |
| 7.1.3.1 | Households | 812.2 | 0.2 | 0.9 | 514.2 | 132.5 |  | 0.2 | 164.0 |
| 7.1.3.2 | Agriculture | 34.1 |  | 20.5 |  |  |  |  | 13.6 |
| 7.1.3.3 | Services | 372.9 | 0.9 | 1.0 | 216.2 |  |  |  | 154.8 |
| 7.2 | Final non-energy consumption | 36.0 | 0.0 | 32.9 |  | 3.1 |  |  |  |
| 7.2.1 | Chemical Industry | 0.0 | 0.0 | 0.0 |  |  |  |  |  |
| 7.2.2 | Other sectors | 36.0 |  | 32.9 |  | 3.1 |  |  |  |

“Energy Statistics Manual”, OECD/IEA, 2007